2018:

[20 marks]

Assuming that the LEDs have been set to some pattern, write MASM code to switch off the LEDs on the diagonals. Your code should process one row at a time.

For example, given the initial pattern:

;

; Data & Code

;

.data

row_num		DWORD	0

mask1		DWORD	080000000h		; from left hand side

mask2		DWORD	01h				; from right hand side

get_patt	BYTE	'Please enter a pattern number: ',0,10

message     BYTE	'Press OK to continue',0

caption     BYTE	'Welcome to CA296',0

.code

main:nop

invoke version

invoke readIntegerWithMessage, ADDR get_patt

invoke setPattern, eax

start:

cmp row_num, 32

je finish

invoke readRow, row_num		; row pattern is stored in eax

from_left:

mov ebx, mask1			; put the mask1 into ebx because we edit it

not ebx					; not mask1 (011111111..)

and eax, ebx			; and our row to the not-ed mask1

from_right:

mov edx, mask2			; put mask2 into edx because we edit it

not edx					; not mask2	(1111111110)

and eax, edx			; and our row to not-ed mask2

turn_off:

invoke writeRow, row_num, eax

reset:

shr mask1, 1

shl mask2, 1

invoke Sleep, 100

inc row_num

jmp start

finish:

invoke MessageBox,0,ADDR message,ADDR caption,MB_OK

invoke ExitProcess,0

end main

When you ‘and’ something you only get a 1 if there are two ones, otherwise it is a zero. Here, because we want to switch lights off, we made our mask be a zero so when we ‘and’ it to the pattern, it can’t return a 1, therefore switches off. (See google doc title ‘Helpful Explanations for Logical Instructions’ for more)

2017:

[ 20 Marks ]

Given the definitions

.data

row DWORD ?

start DWORD ?

finish DWORD ?

assuming that the values of row, start and finish are each in the interval [0..31], write

MASM code to switch off the LEDs in the interval [start..finish] on row row of the

displayed LED pattern.

For example, if row is 13, start is 5 and finish is 8, your code should change the

pattern

;

; Data & Code

;

.data

row_num			DWORD	0

start_point		DWORD	0

end_point		DWORD	0

mask1			DWORD	01h

count			DWORD	0

get_patt		BYTE	'Please enter a pattern number: ', 0

get_row			BYTE	'Please enter a row number to edit: ',0

get_start		BYTE	'Please enter a column off start number: ',0

get_end			BYTE	'Please enter a column off end number: ',0

message		    BYTE	'Press OK to continue',0

caption			BYTE	'Welcome to CA296',0

.code

main:nop

invoke version

setup:

invoke readIntegerWithMessage, ADDR get_row

mov row_num, eax

invoke readIntegerWithMessage, ADDR get_start

mov start_point, eax

invoke readIntegerWithMessage, ADDR get_end

mov end_point, eax

invoke readIntegerWithMessage, ADDR get_patt

invoke setPattern, eax

mov ecx, 0				; my counter (needs to be register to compare)

set_mask:

cmp ecx, start_point		; compare counter to start point

je start

shl mask1, 1

inc ecx						; increase counter

jmp set_mask

start:

cmp ecx, end_point			; compare counter with end point

jg finish					; when greater, we are done

mov count, ecx				; move ecx into counter because ‘invoke’ resets registers

invoke readRow, row_num		; read in the chosen row (then stored in eax)

mov ebx, mask1				; move mask into ebx (to edit we put in a register)

not ebx						; puts 0s where 1s and 1s where 0s

and eax, ebx				; only get a 1 if two ones

invoke writeRow, row_num, eax

reset:

shl mask1, 1

invoke Sleep, 200

mov ecx, count				; puts count back into ecx

inc ecx						; increase counter

jmp start					; go around again

finish:

invoke MessageBox,0,ADDR message,ADDR caption,MB_OK

invoke ExitProcess,0

end main

2017R:

[ 20 Marks ]

; Given the definitions

; .data

; column DWORD ?

; start DWORD ?

; finish DWORD ?

; assuming that the values column, start and finish are each in the interval [0...31],

; write MASM code to switch off the LEDs in the interval [start..finish] on column

; column of the displayed LED pattern.

; For example, if column is 5, start is 5 and finish is 8, your code should change the

; pattern:

.data

col			DWORD	0

start_p		DWORD	0

end_p		DWORD	0

count		DWORD	0

mask1		DWORD	01h

get_start	BYTE	'Please enter a start point',0

get_end		BYTE	'Please enter an end point',0

get_col		BYTE	'Please enter a column number',0

get_patt	BYTE	'Please enter a pattern number',0

message     BYTE	'Press OK to continue',0

caption     BYTE	'Welcome to CA296',0

.code

main:nop

setup:

invoke readIntegerWithMessage, ADDR get_col

mov col, eax

invoke readIntegerWithMessage, ADDR get_start

mov start_p, eax

invoke readIntegerWithMessage, ADDR get_end

mov end_p, eax

invoke readIntegerWithMessage, ADDR get_patt

invoke setPattern, eax

move_mask:

mov eax, mask1		; put mask into eax

sal eax, 1			; move mask (in eax) left 1

mov mask1, eax		; put edited mask into mask variable

inc count			; increase count with each edit

mov edx, count		; put our count into edx (we need one register when comparing)

cmp edx, col		; check if we are in the right place

jl move_mask		; if less, go around again

; if more go to next step

lights_off:

mov eax, end_p		; put our end point into eax

cmp start_p, eax		; check where we are

jg	finish			; we are done when greater

invoke readRow, start_p	; read first row in (stored in eax)

not eax				; turns 111 to 000

mov ebx, mask1		; puts our mask into ebx

or eax, ebx			; forces a 1 to be present

; (you can only get a 0 if two 0s, but our mask is 1)

not eax				; turns our row back to original plus mask

invoke writeRow, start_p, eax

invoke Sleep, 200

inc start_p			; to get to next row

jmp lights_off

finish:

invoke MessageBox,0,ADDR message,ADDR caption,MB_OK

invoke ExitProcess,0

end main

2016:

[ 20 Marks ]

; Given a 32-bit value we can swap its upper and lower 16 bits. For example, given

;  0000 0001 0010 0011 0100 0101 0110 0111

; we can swap its upper and lower 16 bits to get the value:

;  0100 0101 0110 0111 0000 0001 0010 0011

; Write MSAM code to input an integer N in the interval [0...5] and output the LED pattern

; for N with the upper and lower 16 bits of the value for each row swapped. For example,

; given the initial pattern

.data

row_num		DWORD	0

left		DWORD	0

right		DWORD	0

row			DWORD	0

get_patt	BYTE	'Please enter a pattern number',0

message     BYTE	'Press OK to continue',0

caption     BYTE	'Welcome to CA296',0

.code

main:nop

invoke version

invoke readIntegerWithMessage, ADDR get_patt

invoke setPattern, eax

swap_patt:

cmp row_num, 32			; at the end of the rows we are done

je finish

invoke readRow, row_num		; read this row (stored in eax)

mov row, eax				; put eax into variable called row

mov ecx, 16				; put 16 into ecx (then we can call 16 using CL)

shl eax, CL				; shift row left by 16

mov left, eax			; put new row into left

mov eax, row				; move original row into eax

shr eax, CL				; shift right by 16

mov right, eax			; move edited row into right

add eax, left			; add right(eax) and left (answer stored in eax)

invoke writeRow, row_num, eax

invoke Sleep, 200

inc row_num				; next row

jmp swap_patt			; go around again

finish:

invoke MessageBox,0,ADDR message,ADDR caption,MB_OK

invoke ExitProcess,0

end main

2016R:

[ 20 Marks ]

;

; Write MASM code to input an integer N in the interval [0..5] and output the LED pattern

; for N in which only the LEDs that are part of a vertical line from top to bottom in the

; original pattern are left switched on. For example, given the initial pattern

;

.data

on_count	DWORD	0

row_num		DWORD	0

col_num		DWORD	0

mask1		DWORD	0ffffffffh			; we start our mask off with all 1s

get_patt	BYTE	'Please enter a pattern number',0

message     BYTE	'Press OK to continue',0

caption     BYTE	'Welcome to CA296',0

.code

main:nop

invoke version

invoke readIntegerWithMessage, ADDR get_patt

invoke setPattern, eax

make_mask:

cmp row_num, 32			; see how far we are

je reset				; if done go to next step

invoke readRow, row_num		; read in the row (stored in eax)

and eax, mask1			; and our mask (only get 1 if two 1s)

mov mask1, eax			; put result into mask to replace old mask

inc row_num				; next row

jmp make_mask

reset:

mov row_num, 0			; reset row_num

write_patt:

cmp row_num, 32			; see how far we are

je finish				; if 32 - done

invoke writeRow, row_num, mask1		; each row the same as our latest mask

invoke Sleep, 200

inc row_num

jmp write_patt			; go around again

finish:

invoke MessageBox,0,ADDR message,ADDR caption,MB_OK

invoke ExitProcess,0

end main

2016 SAMPLE EXAM:

[ 20 Marks ]

; Write MASM code to input an integer N in the interval [0..5] and output the LED pattern

; for N with all the LEDs inverted. For example, given the initial pattern

.data

row_num		DWORD	0

get_patt	BYTE	'Please enter a pattern number',0

message     BYTE	'Press OK to continue',0

caption     BYTE	'Welcome to CA296',0

.code

main:nop

set_up:

invoke version

invoke readIntegerWithMessage, ADDR get_patt

invoke setPattern, eax

invert:

cmp row_num, 32

je finish

invoke readRow, row_num 		; stored in eax

not eax				; inverts the row

invoke writeRow, row_num, eax	 ; print inverted row

invoke Sleep, 200

inc row_num				; next row

jmp invert				; go around again

finish:

invoke MessageBox,0,ADDR message,ADDR caption,MB_OK

invoke ExitProcess,0

end main